# ECU Run-Time Monitoring System

1. PIC16F887 Microcontroller

- It controls the entire system (brain of the project)

- Handles timer, interrupts, EEPROM, LCD, buttons, LEDs, RTC communication

- Best for low-cost embedded projects

Features

- Controls FAN ON/OFF

- Measures runtime in HH: MM: SS

- Uses Timer1 interrupt to count time

- Reads/writes EEPROM

- Drives the LCD

- Reads keypad inputs

- Reads RTC via I²C

2. 16×2 LCD Display

- To show live data on screen

- User-friendly output device

Features

- Displays FAN timer

- Displays RTC clock (Real Time Clock)

- Shows SET TIME menu

- Shows each digit that the user enters via keypad

3. DS1307 RTC (Real-Time Clock)

- Keeps accurate real time (even after power OFF)

- Internally built crystal + backup battery support

Features

- I²C communication with PIC

- Stores accurate HH: MM: SS

- User can set time via keypad

- Displays current time on LCD

4. Matrix Keypad (Phone Dial Keypad 4x3)

- To input hours, minutes, seconds

- To enter commands like * and #

Features

- Takes digit-by-digit user input

- Used for:

  - Entering HH

  - Entering MM

  - Entering SS

  - Confirming separators (*)

  - Ending input (#)

5. Internal EEPROM of PIC16F887

- To save the FAN total runtime permanently

- Data will not erase after power off

Features

- Stores:

  - Hour

  - Minute

  - Seconds

- When fan turns OFF, time is saved

- When turned ON again, previous time is restored

6. Timer1 Module (PIC Timer Interrupt)

- To count 1 second accurately

- Best for real-time tasks

Features

- Timer interrupt runs every 0.5 sec

- 2 interrupts = 1 second

- Updates sec, min, hour variables

7. Push Button – FAN ON/OFF switch

- To control fan ON and OFF

- Single toggle button

Features

- 1st press → FAN ON, start counting

- 2nd press → FAN OFF, save time to EEPROM

8. LEDs (RA4 & RA5)

- To show visual indication

- Alternating blinking pattern

Features

- First 4 seconds → Both LEDs ON

- From 4th second onward → RA4, RA5 alternate

  - 1 sec RA4

  - 1 sec RA5

  - Repeats

9. 5V Power Supply

- To power microcontroller, LCD, RTC

Features

- Stable 5V regulated supply

- Required for accurate RTC and PIC operation

Full Project Summary

ECU Real-Time Monitoring System

This project monitors and records the total ON time of a fan/engine component similar to an ECU (Electronic Control Unit) runtime counter.
The system displays real time using an RTC and stores runtime permanently using EEPROM.

Automatic Features

- Fan ON → Runtime starts

- Fan OFF → Time saved

- Next ON → Continues from last saved time

- RTC always shows current clock

- LED pattern changes after 4 seconds

- Keypad-based time setting

- LCD shows both FAN-time & RTC-time